Hardware Design Document

FOR

Video Display Unit B3

- Revisions

| Rev No | Date | Changed Item | Pages |
| --- | --- | --- | --- |
| 01 | 11 Nov. 2021 | Initial release of design document |  |

TABLE OF CONTENTS

1	SCOPE	5

1.1	Identification	5

1.2	System Overview	5

1.3	Document Overview	12

2	Definitions, abbreviations and referenced documents	13

2.1	Definitions	13

2.2	Abbreviations	13

2.3	Applicable Documents	14

3	System Hardware	16

3.1	System Definition	16

3.2	VDUB3 data / control overview	19

3.3	System Architecture Overview	20

3.4	SoM UltraScale MPSoC PCBA	21

3.4.1	Hardware features of SoM UltraScale MPSoC PCBA	21

3.4.2	Functions and characteristics of SoM UltraScale MPSoC PCBA	22

3.4.3	Interfaces Definition & control data structure	25

3.5	LCD	27

3.6	Main PCBA (MAIN_PCBA)	27

3.6.1	Power supply	27

3.6.2	Interface	28

3.7	Mechanical Design	28

3.8	Thermal Design	28

3.9	System Characteristics	28

3.9.1	System Performance Characteristics	30

3.9.2	Storage	30

3.9.3	Transport	30

3.9.4	Preparation for Operation	30

3.9.5	Task Performance Times	30

3.9.5.1	Assumptions	30

3.9.6	Operation under normal operation mode	32

3.9.6.1	General functional flow chart	32

3.9.7	Built-in Test	39

3.9.8	Protection against secondary effect of initial cause	41

3.10	System operating status information recording and time synchronization	42

3.10.1.1	Functional Check / Failure Detection and Isolation (offline)	43

3.11	Interface Requirements	44

3.11.1	The requirements of mechanical interface of VDUB3 box	44

3.11.2	Electrical interface between vehicle electronic devices and VDUB3 box	44

3.11.3	System testing / diagnosis / programming	44

3.11.4	Program interface	45

3.11.5	Testing / Diagnosis interface	45

# SCOPE

This document stipulates and defines the system hardware architecture and data structure of the Video Display Unit B3 System (abbreviated as VDUB3 hereafter). It includes the system hardware architecture, internal data structure, interface data exchange model and hardware performance assessment.

The design shall be complied with the requirements specified in the document Video SSRS REQ-328-JH-0005-A_REV_3 <Vision System Requirement Specifications> dated on 5 Nov. 2021.

## Identification

This design document defines the system hardware architecture, system data structure, interface data exchange model for:

Video Display Unit B3

Hereinafter also being referred to as VDUB3.

The objective of this document is to provide the design guideline in the actual design, development, prototyping phase. The guideline proposed hereafter should be implemented in the system design on mechanical / circuit / PCB / software / interfaces aspects.

## System Overview

VDUB3 is a Video Display Unit designed for local situation awareness (LSA), closed hatch driving (CHD) and respective interfacing capabilities, as specified in the document Video SSRS REQ-328-JH-0005-A_REV_3 <Vision System Requirement Specifications>.

Table 1.	Vision System Requirement Specifications

| S/N | DESCRIPTION | STATUS | VERIFICATION METHOD | VERIFICATION DOCUMENT | SRS REF |
| --- | --- | --- | --- | --- | --- |
| 1 | STATES & MODES REQUIREMENTS |  |  |  |  |
| 1.1 | Configuration |  |  |  |  |
| 1.1.1 | DAY DRIVING MODE: Rear view of the vehicle will show on the VDU NIGHT DRIVING MODE: Driver can pull down the VDU and switch to TI mode, where the video feed from the front TI will be shown on the VDU. The driver may choose to turn off the IR illuminator during tactical operations REVERSE MODE: VDU will automatically switch to show video feed from the rear far camera (default) or rear near camera |  | Test (T) | Acceptance Test Plan (ATP) Acceptance Test Report (ATR) |  |
| 1.2 | Operational State (vehicle power on) |  |  |  |  |
| 1.2.1 | The Video Display Unit (VCU) and Vehicle Display Unit (VDU) shall turn on when the vehicle is powered on. The VCU shall supply 12VDC power to the cameras, which shall also turn on. The VCU shall receive SDI video from the cameras and weapon video system and process the video signals and overlay any necessary on-screen display (OSD) information. The VCU shall then output the resultant video signal in HD-SDI format to the VDU and to the C2 computer systems. The IR illuminator shall be controlled by function buttons on the VDU. |  | Test (T) | Acceptance Test Plan (ATP) Acceptance Test Report (ATR) |  |
| 1.2.2 | The Vision System shall provide capacities such as night driving, situational awareness of the external environment and weapon vision to the driver and crew stations |  | Test (T) | Acceptance Test Plan (ATP) Acceptance Test Report (ATR) |  |
| 2 | FUNCTIONAL & PERFORMANCE REQUIREMENTS |  |  |  |  |
| 2.1 | The vision system shall automatically turn on when the vehicle is powered |  | Test (T) | Acceptance Test Plan (ATP) Acceptance Test Report (ATR) |  |
| 2.2 | Common video connector type and pin configuration shall be adopted to enable a common video data connection |  | Inspection (I) | Certificate of Conformance |  |
| 2.3 | The latency incurred for video processing, formatting and distribution shall not affect real time driving and weapon engagement performance (<150ms total) |  | Test (T) | Acceptance Test Plan (ATP) Acceptance Test Report (ATR) |  |
| 2.4 | For the VCU requirements, please refer to the "VCU" tab |  |  |  |  |
| 2.5 | For the VDU requirements, please refer to the "VDU" tab |  |  |  |  |
| 2.6 | For the TI requirements, please refer to the "TI" tab |  |  |  |  |
| 2.7 | For day cameras requirements, please refer to the "Day Cameras" tab |  |  |  |  |
| 2.8 | For the IR illuminator requirements, please refer to the "IR Illuminator" tab |  |  |  |  |
| 3 | EXTERNAL INTERFACE REQUIREMENTS |  |  |  |  |
| 3.1 | Mechanical |  |  |  |  |
| 3.1.1 | The vision system components housed within the vehicle (VDU and VCU) shall have an IP rating of IP65 minimum. The vision system components mounted on the exterior of the vehicle (TI, rear cameras, and IR illuminator) shall meet IP66 minimum. |  | Test (T) | MIL-STD 810G Test report |  |
| 3.2 | Electrical |  |  |  |  |
| 3.2.1 | The Vision System components shall be fully functional within the operational limits of MIL-STD 1275E |  | Test (T) | MIL-STD 1275E test report |  |
| 3.2.2 | The Vision System components shall have built-in protection against reverse polarity |  | Test (T) | ATP test report |  |
| 3.2.3 | All harnesses shall be of rugged construction and the wire terminations are to be made of high quality connectors with excellent resistance to damage from maintenance activity |  | Inspection (I) | Certificate of Conformance |  |
| 3.2.4 | Video output connector on the VCU shall conform to the Def-Stan 23-09 Ethernet Data Network Connector standardisation: Single channel Ethernet connectors at interface panel shall be of type MIL-DTL-38999 Series III Rev L Amdt 1(07/2009), D38999/XXαB35SA |  | Inspection (I) | Certificate of Conformance |  |
| 3.2.5 | All SDI video signal contacts shall use coaxial contacts. All HD-SDI contacts shall be size 12, using 603039 and 603052 for coaxial pins and sockets respectively. |  | Inspection (I) | Certificate of Conformance |  |
| 3.3 | Hydraulic |  |  |  |  |
| 3.3.1 | Not applicable |  |  |  |  |
| 3.4 | Software |  |  |  |  |
| 3.4.1 | The software shall be compliant to MIL-STD 882E |  | Test (T) | SWAT test report White box test report Black box test report |  |
| 3.4.2 | Any customization to the existing Off-the-shelf system to meet STELS requirements is preferred not to have any change to the system’s original software code. Change of parameters is allowable. |  | Test (T) |  |  |
| 4 | ENVIRONMENTAL REQUIREMENTS |  |  |  |  |
| 4.1 | All tests shall meet EQT and ESS requirements |  | Test (T) | MIL-STD 810G Test report |  |
| 4.1.1 | Refer to the following documents. |  |  |  |  |
| 5 | EMI & EMC and POWER REQUIREMENTS |  |  |  |  |
| 5.1 | Contractor shall fulfill the Ebox meet the MIL-STD-461F |  |  |  |  |
| 5.2 | Refer to EMI EMC requirements documents. |  | Test (T) | MIL-STD 461F Test report |  |
| 5.3 | Contractor shall fulfill the Ebox meet the MIL-STD-1275E |  |  |  |  |
| 5.4 | Refer to Power requirements documents. |  |  |  |  |
| 6 | PHYSICAL REQUIREMENTS |  |  |  |  |
| 6.1 | Hardware dimension(s) |  |  |  |  |
| 6.1.1 | Refer to each component's tab for its specifications |  | Inspection(I) | Certificate of Conformance |  |
| 6.2 | Hardware Weight(s) |  |  |  |  |
| 6.2.1 | Refer to each component's tab for its specifications |  | Inspection(I) | Certificate of Conformance |  |
| 6.3 | Paint |  |  |  |  |
| 6.3.1 | The components, which require painting, shall be prepared and painted in accordance with ME-TG-2020-004 |  | Inspection(I) | Certificate of Conformance |  |
| 6.3.2 | The painting shall be done using heat resistant and fire retardant type paint. |  | Inspection(I) | Certificate of Conformance |  |
| 6.3.3 | All slding parts and operation contact surfaces shall be free of paint. |  | Inspection(I) | Certificate of Conformance |  |
| 6.3.4 | The quality of the painting work shall meet the military or industrial requirements in terms of thickness, homogeneity and adhesiveness. |  | Inspection(I) | Certificate of Conformance |  |
| 7 | RESOURCE |  |  |  |  |
| 7.1 | The Vision System Components shall meet IPC 610 and IPC 620 Class 3 product acceptance criteria |  | Inspection(I) | Certificate of Conformance |  |
| 8 | SAFETY REQUIREMENTS |  |  |  |  |
| 8.1 | All material used in the construction of the Vision System Components shall be fire retardant |  | Inspection(I) | Certificate of Conformance |  |
| 9 | SYSTEM SECURITY |  |  |  |  |
| 9.1 | Refer to cybersecurity documents (Pending for Attachment) Conformance to ISO 28000 |  |  |  |  |
| 10 | RELIABILITY REQUIREMENTS |  |  |  |  |
| 10 | RELIABILITY & MAINTAINABILITY REQUIREMENTS |  |  |  |  |
| 10.1 | Refer to R&M documents. |  |  |  |  |
| 10.2 | Minimum Guaranteed MKBF/MTBF of Refer to MTBF figures on individual tabs |  |  |  |  |
| 11 | ILS REQUIREMENTS |  |  |  |  |
| 11.1 | Refer to ILS documents. |  |  |  |  |
| 12 | TRANSPORTABILITY |  |  |  |  |
| 12.1 | Refer to Supplier Quality Requirement document on Other Qualities (No 13.1) |  |  |  |  |
| 12.2 | Refer to ILS Requirement document on ILS Requirements (No 11.1) |  |  |  |  |
| 12.2 | Refer to ILS document |  |  |  |  |
| 13 | OTHER QUALITIES |  |  |  |  |
| 13.1 | Refer to Supplier Quality Requirement document. |  |  |  |  |
| 13.2 | The contractor shall provide a 12 months warrany against manufacturer defeats and workmanshop with the option of 2 yeas extension. |  |  |  |  |
| 13.3 | The contractor shall provide a Certificate of Conformance(COC) and test reports. |  |  |  |  |
| 13.4 | Unless otherwise specified in the contract/purchasing order or applicable drawing, the workmanship and acceptance requirements for the manufacturing of Printed Circuit Board (“PCB”) shall conform to the requirements specified in IPC-A-600, Class 3 product acceptance criteria. |  |  |  |  |
| 13.5 | Unless otherwise specified in the contract / purchasing order or applicable drawing, the workmanship and acceptance requirements for the manufacturing of electrical and electronic assemblies shall conform to the requirements specified in IPC-A-610, Class 3 product acceptance criteria. |  |  |  |  |
| 13.6 | Unless otherwise specified in the contract/purchasing order or applicable drawing, the workmanship and acceptance requirements for the manufacturing of cable and wire harness assemblies shall conform to the requirements in IPC/WHMA-A-620, Class 3 product acceptance criteria |  |  |  |  |
| 14 | Export Control |  |  |  |  |
| 14.1 | Supplier should indicate if Export Control (EUC / EUS) is required |  |  |  |  |

The VDUB3 box external connection diagram is illustrated in figure 1 below.

Figure 1.	System Architecture, VDUB3 External Connection Diagram.

## Document Overview

The contents of this document is divided into following six chapters:

Chapter 1.	Identifies and describes the specified system

Chapter 2.	Lists definition of terms and relevant abbreviations contained in this document and all referenced documents.

Chapter 3.	Specifies all requirements of system.

Chapter 4.	Design & Quality Assurance (QA) considerations.

Chapter 5.	Assign delivery preparations.

Chapter 6.	Contains additional product features.

# Definitions, abbreviations and referenced documents

## Definitions

The following definitions are used in this document:

- Failure:	A failure is defined as the instantaneous termination of ability of an item or equipment to perform its required function.

- Growth Potential:	As an option, but can be realized if a considerable development effort has been undertaken.

- LRU:	A Line Replaceable Unit (LRU) is a hardware that may be subject to replacement at one of the maintenance echelons. A LRU may be a repairable assembly/subassembly or a piece of part/component.

- Option: 	Options are functions or units, which can be selected as an alternative or additionally.

- Typical value: 	Corresponds to the performance value achieved in 50% of the cases.

- Validity of the performance Data: The performance data of VDUB3, within the entire working envelope and under operating environmental conditions as specified in the document. Any exception will explicitly be stated.

## Abbreviations

The following abbreviations are used in the document:

- VDUB3: 	Video Display Unit for B3 project, box.

- PCB:	Printed Circuit Board.

- BIT:	Built in test.

- MMI:	Man-Machine Interface.

- LRU:	Line Replaceable Unit.

- N/A:	Not Applicable.

- GTP:	Gunning Transceiver with low Power.

- PS:	Power Supply.

- DC:	Direct Current.

- AC:	Alternative Current.

- MTBF:	Mean Time Between Failure.

- MTTR:	Mean Time To Repair.

- PA:	Point Availability.

- QA:	Quality Assurance.

- TBD:	To Be Defined.

- ATP:	Acceptance Test Plan.

- CPU:	Central Processing Unit.

- MC:	Micro-Controller.

- ADC:	Analog-to-Digital Converter.

- DAC:	Digital-to-Analog Converter.

- FPGA:	Field Programmable Gate Array.

- SDI:	Serial Digital Interface for SMPTE video transmission.

- RAM:	Random Accessible Memory

- DDR3 SDRAM:	double data rate type three synchronous dynamic random access memory

- SD:	Standard Definition.

- NV:		Non-Volatile.

- DDP:	Driver's Display Panel.

- LSA:	Local Situation Awareness.

- HD:	High Definition.

## Applicable Documents

The Following documents are applicable for the project VDUB3:

- STANAG 2895	Extreme climatic conditions and derived conditions for use in design/test criteria for NATO force.           Material.

- MIL-STD-810G	Test Method standard for Environmental Engineering Considerations and laboratory Test.

- MIL-STD-961D	Department of Defence Standard Practice, Defence Specifications. 				       Material.

- MIL-STD-461F	Electromagnetic Emission and Susceptibility Requirements for control of Electromagnetic Interference. Material.

- MIL-STD-1472D	Human Engineering Design Criteria for Military System, Equipment and Facilities.			           Material.

- MIL-HDBK-454	General Guideline for Electronic equipment.

- ISO-9001	Quality System Model for Quality Assurance in Design, Development, Production, Installation and Servicing.

- MIL-HDBK-217	Reliability Prediction for Electronic System

- MIL-HDBK–338	Reliability Handbook.

- MIL-STD-1629	FMECA Handbook.

- MIL-STD-882E	Department of defence standard practice, system safety.

- MIL-STD-498	Software development and documentation

- MIL-STD-1275E	Characteristics of 28 Volt DC Electrical Systems in Military Vehicles.

- IEC-61000-4-2 	International Standard Testing and Measurement Techniques.

- MIL-STD-831	Test Reports Preparation.

- MIL-STD-1686 	Electrostatic Discharge Control Program for Protection of Electrical and Electronic Parts, Assemblies and Equipment.

- MIL-STD-188-124 	Grounding, Bonding and Shielding for common long haul/tactical communication system including ground base communication, electronic facility and equipment.

# System Hardware

In this chapter, the system electronic / data / mechanical architecture would be defined. The system shall be built to fulfil the requirements.

## System Definition

The system shall be designed with the functional modules illustrated as in below figure 2.

Figure 2.	VDUB3 System functional modules block diagram

The basic features / components / sub-systems diagram in Figure 2 shows the VDUB3 box may consists of:

- Main PCB, which connects external I/Os and internal circuitry

- Connection to external panel connectors for

  - 1x HD-SDI + 3x HD/SD-SDI(optional) input, with input conditional circuit to Video Processing module.

  - 1x GbE PHY driver between external panel connector and video processing module.

  - 1x RS422 serial port drivers between the video processing module.

  - 9x DI between push buttons and the video processing module.

  - LVDS and USB2.0 connection between LCD and the video processing module.

  - 1x DC/DC module, convert the vehicle power supply for internal use, 12VDC@30W

- Connection with video processing module

- Video Processing module, SoM PCBA based on Xilinx UltraScale MPSoC.

The 2 functional modules are designed to be placed in one metal housing box with integration of external connectors / functional PCBs / software. The VDUB3 box shall be designed for:

- Use within a ground mobile environment

- Fast set-up in a vehicle deployment

- Input 4x channels of SDI SMPTE digital video streams (up to HD-SDI).

- Capability to detect the existence of input video signals in 3 above.

- Display video on LCD in low latency.

- Generate OSD upon request.

- Screen tuning for brightness, contrast, colour upon button operation.

- Transmit/receive command via RS422 upon button operation.

- H.264/H.265 video streamer based RTP for 8 encoders and 8 decoders. (optional)

- Video stitching upon request. (optional)

- Video fusion upon request. (optional)

- Motion detection upon request. (optional)

- Automatic failure detection and localisation (BIT)

- High failure detection and localisation probability

- Operational reliability

- Operational safety

- High reliability, even under adverse conditions

- Low MTTR

- Optimal cost / performance ratio

- Low life cycle costs

- Minimal personnel requirements for operation and maintenance

- In-system programmable in engineering mode.

- In-system configuration in Engineering mode.

- Engineering support for user application development.

## VDUB3 data / control overview

- The VDUB3 performs to process the incoming video signals, and display on LCD with OSD.  The data / control illustrated in figure 3 below.

Figure 3.	Data / control information Structure of VDUB3

## System Architecture Overview

- Based on the system data / control, I/O model and functional requirements specified in previous chapters. An overall structure of the system is developed as below.

Figure 4.	VDUB3 System architecture

As in above figure, the system is designed within an Aluminium housing, with external heatsink. The box thermal design is using thermal fins, fanless cooling.

- The SoM UltraScale MPSoC PCBA, which is directly mounted onto the rear cover of the VDUB3 Box, functions as video processing module in figure 4. It is a video processing module, with GbE, Serial I/O.

- Main PCBA, is a mother board for SoM MPSoC PCBA. There is a EMI fitter module and a DC/DC module in figure 4. It provides +12VDC @30W for internal use. The PCBA also contains the drivers for SDI input, GbE, RS422/RS232, LVDS, USB2.0, DIOs etc.

- Internal harness IH1/IH2/IH3/IH4/IH5/IH6.

- 7" 1920 x 1080 Touch Screen LCD.

- 9 push buttons.

## SoM UltraScale MPSoC PCBA

It contains a MPSoC of Quad-core ARM® Cortex-A53 and Dual-core Cortex-R5 Real-time Processors Combination of Mali-400 MP2 GPU and 16nm FinFET+ Programmable Logic Integrated H.264 / H.265 video codec, codec up to 4Kx2K (60fps) video.

### Hardware features of SoM UltraScale MPSoC PCBA

Table 2.	Major features / parameters of SoM MPSoC PCBA (ACV4EV-Teamone)

| SN | Feature | Description | Parameters |
| --- | --- | --- | --- |
| 1 | Core CPU | Quad ARM Cortex A53 | Main Frequency 1.2GHz |
|  | Real-time MCU | Dual ARM Cortex- R5 | Clock 500 MHz |
| 2 | PS RAM | DDR4 512M x 64 Bit | 2400Mbps |
|  | PL RAM | DDR4 512M x 16 Bit | 2400Mbps |
| 3 | GPU | Mali-MP-400 MP2 | 667MHz |
| 4 | Speed Grade | FPGA clock | -1 |
|  | Working Temperature | Industrial Grade | -40°C ~ +85°C |
| 5 | PS side high speed connection | PCIe Gen2 x4 | 500Mbyte/s/lane x 4 lanes |
|  |  | USB3.0 x2 | 2.5Gbps / port x 2 ports |
|  |  | SATA x 1 | SATA3.1 |
|  |  | Display port x1 | DisplayPort |
|  |  | GbE | 4x Tri-mode GbE |
| 6 | FPGA Logic cells | 192k cells | 87.8k LUTs |
|  | FPGA RAMs | Distributed RAM | 2.6Mbits |
|  |  | Block RAM | 4.5Mbits |
|  |  | UltraRAM | 13.5Mbits |
|  | FPGA clock | Global Clock | 4 |
|  | FPGA DSP slices | DSP slices | 728 |
| 7 | EMMC flash | Program/data storage | 8GBytes |
| 8 | QSPI flash | FPGA user data storage | 256MBytes |
| 9 | Video CODEC | H.264/265 CODEC | 8x encoder + 8x Decoder |
| 10 | PL GTH Transceiver | GTH transceiver | 12.5Gb/s/lane x 4 lanes |
| 11 | FPGA HP I/O | High performance I/O | 96 |
| 12 | FPGA HD I/O | High range I/O | 84 |
| 13 | Power supply | Power input | +12VDC @ 30Watt |
| 14 | Size | Dimension | 80mm x 60mm x 7.1mm |
| 15 | Mounting holes | Dimension | M3 x 4 |

### Functions and characteristics of SoM UltraScale MPSoC PCBA

The below table shows the functions and characteristics of the SoM UltraScale MPSoC PCBA.

Table 3.		SAP BD functions and characteristics based on circuit modular level.

| SN | Functions | Characteristics | I/O | Whereby |
| --- | --- | --- | --- | --- |
| 1 | SDI input interface x4ch GTH0~GTH4 RXD input. | SMPTE format to raw video data conversion with error detection, format detection, auto-adaptive feature (or configurable) x 4ch. ea. real-time operation with <20 lines latency | Input: SDI video from external, in SMPTE format | MPSoC PL side, from SDI equalizer driver. |
| 2 | Video frame read / write | Write / read video frames or feature frames to / from DDR4 RAM, write size adaptive features, error detection features, real-time operation with < 1+ frame delay | Input: frame size / rate, pixel clock, error message, read / write control Output: status message I/O data: Raw video data | In FPGA |
| 3 | Video frame resizing | Resizing part or full image frame to desired size. real-time, latency <1+ frame, 3x3 macroblock bi-linear interpolation or down-sampling. | Input: source frame start position /size, target frame start position /size. source video data Output: target video data, error message | In FPGA, optional |
| 4 | Video stitching | Stitch front / rear 3 (fused or non-fused) video images into 1 big frame, real-time, latency <1+ frame | Input: source video ID, command Output: stitched video frame, video frame size. | In GPU, optional |
| 5 | Video fusion, optional | Fuse daysight camera with corresponding TI camera, real-time, latency <1+ frame | Input: command, source video IDs, alpha-blending parameters. Output: output video frame, error message, video frame size. | In FPGA, optional |
| 6 | Motion detection | To detection the motion using accumulated average background subtraction method. Latency<1s. real-time or not. | Input: command, video ID, sensitivity, scenario. blocked location. Output: motion frame with block on moving targets. | In GPU, optional |
| 7 | OSD generate | To Generate OSD frame as desired. non real-time. | Input: command, OSD filename. Output: OSD frame | FPGA. |
| 8 | OSD superimpose | To superimpose OSD frame onto output video frame, real-time, latency <1frame | Input: output frame data, OSD frame data, alpha-blending rate. Output: Final frame data | FPGA |
| 9 | Video output | Convert video to LVDS format and output to LCD | Input: SDI video input, command, OSD. Output: LVDS video | FPGA |
| 10 | GigE Ethernet TCP/IP management | To manage the commands on Ethernet, according to RTP protocol | TBD | PS |
| 11 | Video CODEC | To H.265/H.264 CODEC, MPEG AV CODEC. | Input: final frame data Output: H.264/H.265, MPEG | FPGA/PS , optional |
| 12 | Command interpretation | To interpret command from touch screen USB2.0 | Input: command/info from touch screen Output: command to CPU | PS |
| 13 | Command interpretation | To interpret command from Ethernet links | Input: command/info from external Output: command to CPU | PS |
| 14 | Command interpretation | To interpret command from UART, or extended RS422 | Input: Commands for control Output: commands to PS RT-MCU | PS |
| 15 | Programming diagnosis | To program / diagnosis / configure | Input: command, program file, data file from external Output: to program FPGAs, CPU or update configuration file etc. | PS/PL |
| 16 | BIT test | To conduct BIT test continuously | As defined | PS |
| 17 | Power up test | To conduct power up test | As defined | PS |
| 18 | System initialization | To initialize system as default | As defined | PS |
| 19 | Fault protection | To protect system from secondary effect damage | TBD | PS |
| 20 | Others | TBD | TBD | PS |

### Interfaces Definition & control data structure

The interfaces on SoM UltraScale MPSoC PCBA could be categorized as in below table.

Table 4.		SoM UltraScale MPSoC PCBA interfaces.

| SN | Functions | Characteristics | Protocol and data | Whereby |
| --- | --- | --- | --- | --- |
| 1 | Interfaces to external devices out of the VDUB3 |  |  |  |
| 1.1 | SDI input / output interface | GTH Rx with extended SDI equalizer. Max. 2.97Gbps | Input: SDI video from external, in SMPTE format | In PL GTH link Rx/Tx, total 4 channels. |
| 1.2 | Gigabit Ethernet | RTP video data | Video, commands, information in RTP protocol | In PS |
|  |  | RTP control / information layer |  |  |
|  |  | TCP/IP Data / message, data file transfer, program file transfer etc. | TBD, in TCP/IP protocol for diagnosis / configuration / programming |  |
| 1.3 | RS422 | External control ports | Commands, information | PS |
| 1.4 | DIO | discrete signal | Switching signals | PS/PL |
| 1.5 | USB2.0 | Linux configuration port | Commands, information in Linux control panel | PS |
| 1.6 | USB2.0 | Touch panel interface | Touch operation from LCD touch panel | PS |
| 1.7 | +12VDC power supply | From power supply PCBA | 12V@30W | MPSOC and all. |

The initial control from the external serial ports / Ethernet commands source is firstly interpreted in the MPSoC PS side, then the command is forwarded to respective functional modules. The relationship between the action and control commands shall be constructed as in figure below.

Figure 5.	Control / configuration data relationship diagram

## LCD

- This module is a 7” FHD of 16:9 format amorphous silicon TFT. The pixel format is vertical stripe and sub pixel are arranged as R(red), G(green), B(blue) sequentially. This display is RoHS compliant, COG(chip on glass) technology and LED backlight are applied on this display.

| - Item | - Characteristics |
| --- | --- |
| - Module dimensions | - 178.76(W)mm x 115.0(H)mm x 11.75(D)mm |
| - LCD Active Area | - 155.52(W)mm x 87.48(H)mm |
| - Pixel Pitch | - 0.081(W)mm x 0.081(H)mm |
| - Resolution | - 1920 x 3(RGB)(W) x 1080(H) dots |
| - Interface | - LVDS |
| - Touch Panel | - Projected Capacitive type |
| - Backlight | - 3 LEDs Series x 10 Parallel |
| - Brightness | - 600cd/m2, typical |
| - Contrast Ratio | - 800, typical |
| - Viewing Angle | - 85º (R) 85º (L) 85º (U) 85º(D), typical |

## Main PCBA (MAIN_PCBA)

- Main PCBA consist of power supply, SDI equalizers, GbE transceiver, and RS422 Transceiver.

### Power supply

- The power circuit consist of:

- +24VDC vehicle power input circuit inclusive of MIL-STD-1275G surge suppression circuit, MIL-STD-461E EMI/EMC filter. current rating at10A@24VDC.

- +12V@30W power supply to internal use.

- Fuses for protection

- Reverse polarity protection.

### Interface

- 4x SDI equalizer

- 1x GbE transceiver

- 1x RS422 Transceiver

## Mechanical Design

## Thermal Design

## System Characteristics

The VDUB3 shall operate in the following main modes:

- Normal operation mode

This main mode is selected for normal operations as default, when the system is properly setup without failure and degradation. Special functions, sub-modes and parameters selectable in other modes and which are not applicable to normal operation mode shall be automatically reset to the default condition or values.

Switching over to another main mode shall be only possible when specific commands are sent from the external device with users’ intervention, or alternatively switches to degraded mode if fault was detected without user's intervention, otherwise the system should be kept in running in normal operating mode. In the background, the Operational Monitoring (OPM) program shall be continuously running to detect the health of the system.

- Degraded (or Emergency) operation mode

This operation mode would be automatically activated if the system detected any fault / degradation from the input or in electronic circuit assembly, or upon intervention from user's request. Under this operation mode, system performs to function

- If the system detected any fault / degradation in the SDI input, alert information shall be sent to the VCU and should display blue screen on LCD, no freeze of past images shall be allowed.

- If the system detected any fault / degradation in the VDU video processing module or on its input, the system displays alert information on LCD if possible, otherwise the LCD should display blue or black screen. No freeze of past images shall be allowed.

- SET UP / Configuration mode (engineering mode 1)

To enter this operating mode, the VDUB3 box shall be connected with special equipment (PCs with configuration software), start this mode with special commands from user interface on the equipment. The mode was performed to setup / configure the VDUB3 units:

- Redefine the image layout for normal operating mode.

- Re-draw the OSD in the engineering field with / without integrated on vehicle.

To exit from the mode, the system need re-boot (with commands or power off-on).

- In-system programming / In-system diagnosis mode (engineering mode 2)

To enter this operating mode, the VDUB3 box shall be connected with special equipment (PCs with programming / diagnosis software), start this mode with special commands from user interface on the equipment. The mode was performed to setup / configure the VDUB3 units:

- Reprogram the system functions for major changes.

- Diagnose the system for testing, monitoring purpose.

To exit from this mode, system need cold start (power off /on).

- TEST functions.

This main mode shall consist of the following sub-modes:

- ORC– Operational Readiness Check, performed during start up.

- FLC – Functional Check, built-in-test running at the background for all modes.

- PFT – Performance Test, performed in engineering mode 2.

- ADJ – Adjustment, performed in engineering mode 1.

### System Performance Characteristics

The performance of the VDUB3 is defined with reference to the following functions:

- Storage

- Transport

- Preparation for Operation

- Operation

- Test Features

### Storage

The VDUB3 shall be able to be stored under the specified environmental conditions, while protected against adverse weather conditions for one year without maintenance.

### Transport

When the VDUB3 is fixed on the base platform such as a vehicle, it shall be transportable together with the vehicle in the infantry battle field.

### Preparation for Operation

The preparations include all handling and performance of functions, which are necessary before the VDUB3 can be brought into operation:

- Deployment

- Connecting of cables if any

- Equipment check

- Power-on

- Set-up & configuration

- Performance Test

### Task Performance Times

#### Assumptions

The specified task performance times are based on the following assumptions:

- The VDUB3 are properly configured.

- The cables are already connected.

- The users are trained

##### Times

##### Time bringing into Operation (Default)

- Bringing the VDUB3 box into operation, i.e. from power supply on, electronics preheat phase (if there is or necessary), equipment check, set-up shall be less than 60 seconds.

- No additional calibration is required.

- Ready for operation if there is no warning or alerting message of failure.

##### Time to enter degraded / Emergency mode

- Ready for operation if there is warning or alerting message for detected failure.

- Respond in less than 1 second to activate the functional mode.

##### Time bringing into configuration / setup mode (engineering mode 1)

- Bringing the VDUB3 box into configuration / setup mode instructed from the connected configuration equipment, i.e. switched from normal operation mode to configuration / setup mode, in less than 10 seconds.

- User's intervention is required in this operation mode.

Ready for operation if there is no warning or alerting message for detected failure.

##### Time bringing into In-system programming/in-system diagnosis (engineering mode 2)

- Bringing the VDUB3 box into operation with connection of programming / diagnosis equipment, i.e. from power supply on, electronics preheat phase (if there is or necessary), equipment check, set-up shall be less than 60 seconds.

- No additional calibration is required.

Ready for operation if there is no warning or alerting message for detected failure.

### Operation under normal operation mode

In normal operation mode situation, on the VDUB3, the following functions or activities are executed:

- Power up initialization. System is boot up and would be set to default configuration as programmed in NV memory.

- VDUB3 shall monitor the command input from different devices, includes:

  - Command input from each individual Ethernet IP / Ports out of the UIT terminals.

  - Command input from VCU RS422.

  - Command input from touch panel USB port.

  - Commands from diagnosis ports.

  - Commands from configuration ports.

- VDUB3 shall respond to generate view / OSD accordingly to commands from the VCU, UIT terminals, or reply to correspond terminal if necessary.

- VDUB3 would enter Programming / diagnosis mode if valid command is received from programming / diagnosis ports.

- VDUB3 shall enter configuration / setup mode if valid command is received from the setup / configuration ports.

- System shall respond to respective terminals as required if invalid commands are received.

- Heart-beating message shall be built in system as required.

- BIT, OPM should be continuously taken place, enter degraded operating mode if non-critical failure was found or enter emergency mode if critical failure was found.

#### General functional flow chart

The operation follow of the VDUB3 is illustrated as in below figure.

Figure 6. Operational flow of VDUB3

##### System power up and initialization

System power up and initialization should be completed in: < 20Sec.

The System power up initialization is done to start the operation of the system hardware and establish external connections with the extended devices / external connections, the functions include:

- FPGA configuration as default from correspondent program flash memory

- CPU core initialization

- OS loading from correspondent flash memory for each ARM core

- Extended device initialization including discrete digital IOs, RS232 / RS422 serial communication ports, SPI / I2C ports, USB ports, ADCs.

- Loading application software from extended application program memory, eMMC.

- Start application software in default setting, to initialize the Ethernet ports.

- Load default communication protocols for Ethernet ports, RS232 / RS422 / SPI / I2C for operation.

- Set the video decoder, video encoder modules as in default setting.

- Start heart beating function for each individual MPSoC.

- Synchronization of system calendar clock.

##### System power up test

System power up test should be completed in: < 20Sec.

The System power up test is done to verify the readiness of each individual module and the inter-connectivity between them. The test includes:

- Test on CPU core and its registers and clocks.

- Test on FPGA configuration and clocks.

- Test on system memory, such as DDR4 SDRAM and Flash memory.

- Test on power supply voltage and current loading

- Test on I/O circuit and functions.

- Test on serial communication ports and settings, such as RS232 / RS422 / SPI / I2C.

- Test on video I/O circuit, such as video decoder and encoder, video signals.

- Test on incoming video stream correctness.

- Test on output video Ethernet port and connectivity.

- Generate heart beating for each individual module as required, through digital signal link (in frequency and duty) or serial link (in control information update)

- Update the test result and refresh system status on relevant memory location.

After power up test the system would enter normal operating mode by default. System is ready for operation hereafter.

##### Normal Operation mode Default Setting

VDUB3 would enter normal operating mode by default regardless of the error found during the power start up test. The system shall be able to automatically enter degraded operating mode according to the error found in the normal operating mode.

In default operating setting the functions could be illustrated as in table below.

Table 5.	Normal Operating mode default function setting.

| ID | Name | Default Setting | Remarks |
| --- | --- | --- | --- |
| 1 | Input SDI video setting | Auto-Detect video format (SMPTE), Auto-configuration for processing. | Or pre-fixed format for individual input channels. |
| 2 | RS422 serial port | 112500bps, 8N1 |  |
| 3 | LCD default screen | 1080P30/P60 | OSD TBD |
| 4 | System Heart Beating information | TBD | Format to be defined. |

##### Control and functions in normal operating mode

In normal operating mode, the VDUB3 functions are controlled through external interfaces.

Table 6.	VDUB3 Control interfaces and functions.

| ID | Functions under control | Source of control | Control message format |
| --- | --- | --- | --- |
| 1 | Video display | Push button, RS422 | VDU send command to VCU via RS422 to change display mode according to push button operation. Protocol is TBD. |
| 2 | Video OSD | Push button, RS422, GbE | VDU generate and display contents and OSD on LCD according to RS422 /Ethernet command. |
| 3 | LCD screen tuning | Push button | VDU tuning brightness, contrast, colour, and NVG-compatible low-brightness setting according to push button operation. |
| 4 | Change view, control IR Illuminator, manual NUC | Push button, RS422, from VCUB3 to cameras | According to the control protocol provided by the camera supplier, to NUT, access the camera. And IR illumination. |
| 5 | VDUB3 programming | From Programming tools, RS232 and Ethernet and JTAG | According to the requirements, to program the functional FPGA modules, ARM cores and CPUs respectively |
| 6 | VDUB3 Configuration | From Configuration Ports on VDUB3, RS232, Ethernet. | To configure the OSD graphical image, preset of the display layout in item 1, 3. |
| 7 | VDUB3 diagnosis | From diagnosis Ports on VDUB3, RS232, Ethernet. | To diagnosis the VDUB3. Or through USB2.0. |

##### VDUB3 Characteristics and functions (normal operating mode)

The VDUB3 box external characteristics and functions could be listed as in table below.

Table 7.	VDUB3 Characteristics and functions

| SN | Category |  |  | Characteristics |  |  |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Physical specifications |  |  |  |  |  |  |  |
| 1.1 | Physical Size |  |  | TBD |  |  |  |  |
| 1.2 | Weight |  |  | TBD |  |  |  |  |
| 1.3 | Mounting dimension |  |  | TBD |  |  |  |  |
| 2 | Electrical specifications |  |  |  |  |  |  |  |
| 2.1 | Input Vehicle Power supply | Operating voltage range |  |  | 18V ~ 32V | MIL-STD-1275E |  |  |
|  |  | Peak Current rating |  |  | <2A for <1sec @ 24VDC |  |  |  |
|  |  | Continuous current rating |  |  | <1A @ 24VDC |  |  |  |
| 3 | Video IO characteristics |  |  |  |  |  |  |  |
| 3.1 | Digital video input x 4 CH. | Resolution | SD video, 720x576pixels@25fps HD video, 1920x1080pixels@30fps All in SMPTE format |  |  |  |  |  |
|  |  | Data format |  |  |  |  |  |  |
|  |  | Frame Rate |  |  |  |  |  |  |
| 3.2 | RTP video stream over IP |  | H.264/H.265 RTP, MPEG video streaming x 8 |  |  | 1920x1080@30fps MISB0604.3 data |  |  |
| 4 | Control IO interfaces characteristics |  |  |  |  |  |  |  |
| 4.1 | Remote VCU control (TBD) |  | RS422 (TBD) |  |  | VDU view selection |  |  |
| 4.2 | Panel discrete input |  | 3.3V/0V switching |  |  | Discrete x 9 |  |  |
| 4.3 | Terminal control link |  | TCP/IP Ethernet vision control layer |  |  | Protocol TBD |  |  |
| 4.4 | Touch screen |  | USB2.0 |  |  | interested zone display, object tracking |  |  |
| 5 | Built-in test and failure detection / isolation / location functions |  |  |  |  |  |  |  |
| 5.1 | BIT functions |  | Power supply malfunction detection. |  |  | BIT test result would be sent to terminals through network and local panel LED would alert. System shall operate appropriately according to the fault detected. |  |  |
|  |  |  | System over-heat detection |  |  |  |  |  |
|  |  |  | Individual module (FPGA modules) malfunction detection |  |  |  |  |  |
|  |  |  | RAM fault detection (frame pixels data checksum counter check, ensure no freeze image) |  |  |  |  |  |
|  |  |  | CPU loading detection |  |  |  |  |  |
|  |  |  | Network loading detection (OSD lamp indicator) |  |  |  |  |  |
|  |  |  | RS232 / RS422 communication malfunction detection |  |  |  |  |  |
|  |  |  | Discrete control signal malfunction detection |  |  |  |  |  |
|  |  |  | Input loss / failure detection (CD detection, & incoming data inspection for ORC, image clear if loss, or serious error) |  |  |  |  |  |
| 6 | System programmability, configuration, monitoring |  |  |  |  |  |  |  |
| 6.1 | CPU (ARM & DSP cores) program update |  | In-system programmable (for application software update only) |  |  | Through Ethernet or RS232 |  |  |
| 6.2 | System debugging |  | TBD |  |  |  |  |  |
| 6.3 | FPGA program update |  | In-system programmable |  |  | Interface TBD |  |  |
| 6.4 | System monitoring |  | In-system real-time monitoring, BIT test result retrieve (sequential period printf) |  |  |  |  |  |
| 6.5 | System fault record |  | Non-volatile memory to record the fault found in system, could be retrieved. |  |  | Interface TBD |  |  |
| 6.6 | System configuration |  | System clock or features could be configured externally |  |  | Interface TBD |  |  |
| 6.7 | System utilize record |  | TBD |  |  |  |  |  |

##### Degraded Operating Mode

The system shall enter degraded operating mode if any fault in system or loss of connection with input / output interfaces.

In this chapter, the degraded mode operating condition and its effects would be defined.

Table 8.	VDU degraded operating mode.

| SN | Fault condition | Operating mode | Effect |
| --- | --- | --- | --- |
| 1 | SDI input lost for display output | degraded mode 1 | - |
| 2 | SDI backup input lost | degraded mode 2 | more delays |
| 3 | Ethernet video stream lost | Mute mode | blue screen |
| 4 | RS422 communication failure | degraded mode 3 | video maybe incorrect |
| 5 | CPU fault | Mute mode | black screen |

### Built-in Test

- Built-in-Test (BIT) shall be periodically conducted at the background during the system operating after operating modes starts. While the BIT is operating, the system is functioning to detect the fault in system hardware and the connectivity between modules or between VDUB3 and external devices. Tests are also conducted to verify the validity of the incoming / outgoing data / information. The operation of BIT shall not induce any performance degradation to the system. Below table shows the functions and characteristics of BIT.

Table 9.	Built-in Test characteristics and functions

| ID | Description | Characteristics & functions | Remarks |
| --- | --- | --- | --- |
| 1 | BIT Characteristics |  |  |
| 1.1 | Conducting Frequency | > 1 time every second |  |
| 1.2 | Testing coverage | > 85% of the hardware modules performance in major functions and settings |  |
| 1.3 | Loading to CPU and system | Shall not affect the functional operation | Put second priority |
| 1.4 | Respond time | Able to find the fault within 5 seconds when occurred |  |
| 2 | SDI input / output |  |  |
| 2.1 | SDI equalizer failure | Defects on video I/O interface | Error code TBD, LED alert, information alert, degraded operating mode activation. |
| 3 | FPGA video processing modules |  |  |
| 3.1 | Power supplies fault | Power loss, power error |  |
| 3.2 | FPGA functional error | ??? |  |
| 3.3 | FPGA and/or data memory error | Memory access error or data readiness / integrity check |  |
| 3.4 | FPGA over temperature | Sense the temperature on each individual FPGA chip and set threshold for fault. |  |
| 3.5 | FPGA input data integrity check | Check the incoming data integrity / format |  |
| 3.6 | Serial comm. connectivity check | Check the connectivity on GTP and parallel ports |  |
| 4 | ARM core and extended IOs |  |  |
| 4.1 | ARM core function registers | Check the major registers |  |
| 4.2 | ARM core loading check | Check the CPU loading |  |
| 4.3 | ARM CPU RAM occupation | Check the used program / data / system RAM memory |  |
| 4.4 | Ethernet network performance | Check the data transmission failure rate, actual data rate and network setting |  |
| 4.5 | USB port check | Check the setting and performance, data integrity and connectivity of the USB port |  |
| 4.6 | UART check | Check the setting, data integrity and connectivity of UARTs |  |
| 4.7 | RS232, RS422 connectivity | Check the connectivity, status of the RS232, RS422 driver. |  |
| 4.8 | I2C/SPI performance | Check the connectivity, status and data integrity of the ports |  |
| 4.9 | SD card and interface | Check the performance, data integrity on SD card |  |
| 4.10 | Digital control signal validity | Check the validity of control signals |  |
| 5 | Ethernet links |  |  |
| 5.1 | Ethernet Connectivity | Check the connectivity of the Ethernet links |  |
| 5.2 | Ethernet BER | Bit error rate on incoming data |  |
| 5.3 | Validity of incoming commands | Check the validity of incoming commands, data readiness check |  |
| 6 | System power supply |  |  |
| 6.1 | System power loading current | Sense the current of supplies |  |
| 6.2 | System power voltage | Sense the voltage of supplies |  |

### Protection against secondary effect of initial cause

Protection hardware or software controlled functions must be built to prevent the system being damaged as secondary effect of the initial inadvertent cause, such as unexpected surges or out-of-specification in voltage or current on input / output links.

Table 10.		Protection features on VDUB3

| ID | Description | Compliance | Remarks |
| --- | --- | --- | --- |
| 1 | Protection on +24V vehicle power input |  |  |
| 1.1 | Surge and Spike protection | MIL-STD-1275D |  |
| 1.2 | Over voltage protection |  |  |
| 1.3 | Over-current (input) protection | Fuse, circuit breaker |  |
| 1.4 | Over loading current protection | Fuse |  |
| 1.5 | Over temperature protection |  |  |
| 1.6 | Output short circuit protection | Complied |  |
| 1.7 | Reverse polarity protection | Complied |  |
| 2 | Video IO link protection |  |  |
| 2.1 | ESD protection on video input link | >8000V ESD Human body model |  |
| 2.2 | Short circuit protection | Complied |  |
| 2.3 | Reverse polarity protection | No damage |  |
| 3 | GigE Vision Ethernet Link |  |  |
| 3.1 | ESD protection on Ethernet link | >8000V ESD Human body model |  |
| 3.2 | Short circuit protection | Complied |  |
| 4 | Other IO ports |  |  |
| 4.1 | All other IO ports such as Ethernet, discrete control signals, RS232, RS422 links shall be protected against ESD | >8000V ESD Human body model |  |
| 4.2 | All other IO ports such as Ethernet, discrete control signals, RS232, RS422 links shall be protected against short circuit connection | Complied |  |
| 4.3 | All other IO ports such as Ethernet, discrete control signals, RS232, RS422 links shall be protected against reverse polarity connection | Complied |  |
| 5 | Protection Critical fault detected |  |  |
| 5.1 | One or multiple FPGA over temperature | Stop the function on fault chip | And record error before system halt |
| 5.2 | Over current sensed on FPGA power | Stop operation |  |
| 5.3 | FPGA programming error | User stops operation | Record error and report |
| 5.4 | DDR4 SDRAM fault | Stop operation | Record error and report |
| .. | ................... | ..................... | ................ |

## System operating status information recording and time synchronization

- System operating status shall be recorded based on the synchronized system clock.

- The system calendar clock shall be synchronized when system start if the connection between VDUB3 and any one of the UIT terminals could be established, based on a request send from VDUB3 when it is powered up. If the connection could not be setup within 10 seconds, the system shall take default calendar time for recording. The information is record for utilize tracing and trouble-shooting purpose. The engineer could also evaluate the reliability of the system.

Table 11.	System information historical records

| ID | Description | Characteristics | Remarks |
| --- | --- | --- | --- |
| 1 | System Total Operating time | Accuracy to 1 minute for each time when system is on. |  |
| 2 | System total power on/off times | <+/- 1 time |  |
| 3 | System error record | Time stapled error record when the error code is changed | Every time upon power up |
| 4 | System operational history | Optional based on setting | System IO, user input, commands input. |
| 5 | Maximum records of error | > 10k |  |
| 6 | History records maximum number | >10k |  |

#### Functional Check / Failure Detection and Isolation (offline)

The Functional Check shall be used for the complete test of all functions of the VDUB3, including those test items that could not be performed by BIT. It is carried out in less than 120 minutes. During the Functional check, the VDUB3 functions are allowed to be interrupted. The Functional Check is started by the commands from system CPU with aid of the external test simulators or equipment. For some tests the operator takes an active part in the functional check by performing steps manually.

Failure Isolation is part of the functional check. It identifies the defective LRUs for replacement.

The Performance Test is used for preventive maintenance or design acceptance test. The Performance Test checks the performance data of functions which cannot be detected by the functional checks. The Performance Test procedure indicates to the specialist what steps are to be taken if the measured values lie outside specified limits.

For some Performance Tests special measuring equipment is used. During the Performance Test the tactical operation is interrupted.

The performance test shall include the test on:

- System responding time measurement

- CPU performance measurement

- External IO connection performance test

- Others if any

## Interface Requirements

- The main interfaces of the VDUB3 unit shall be defined in the document VDUB3-ICD-xx. The interfaces will consist of:

- Mechanical interface between the VDUB3 box and vehicle platform.

- Electrical interface between in-vehicle electronic devices and VDUB3 box.

- Electrical interface between VDUB3 box and testing / diagnosis / programming equipment.

### The requirements of mechanical interface of VDUB3 box

- The VDUB3 shall be installed on the vehicle platform with 4 screws mounting. The design of the VDUB3 mounting interface shall be compatible with MIL-STD-810. Additional earthling stripe shall be built on mounting interface for earthling purpose.

### Electrical interface between vehicle electronic devices and VDUB3 box

- VDUB3 external electrical interface shall be used for connection between the VDUB3 and vehicle electronic devices. The design / fabrication of the interfaces shall comply to the environmental condition (MIL-STD-810) and electro-magnetic requirements (MIL-STD-461E), based on the compliance with the electronic transmission requirements for each individual signal path.

### System testing / diagnosis / programming

The VDUB3 should be design with the In-Field-Program feature (IFP).

### Program interface

In-Field-Program interfaces are implemented with programming ports such as JTAG / RS232 / Ethernet /USB. The function can be started only when the valid special program device is connected. It is option that if the interface can have the feature to debug the system real time. Program interface normally is offline link.

### Testing / Diagnosis interface

This interface is implemented with ports such as JTAG / RS232 / Ethernet. The function can be started only when the valid special program device is connected. It is option that if the interface can have the feature to debug the system real time. Testing / diagnosis interface normally is offline link.

Annex SSRS-Vision

| S/N | DESCRIPTION | STATUS | VERIFICATION METHOD | VERIFICATION DOCUMENT |
| --- | --- | --- | --- | --- |
| 1 | STATES & MODES REQUIREMENTS |  |  |  |
| 1.1 | Configuration |  |  |  |
| 1.1.1 | DAY DRIVING MODE: Rear view of the vehicle will show on the VDU NIGHT DRIVING MODE: Driver can pull down the VDU and switch to TI mode, where the video feed from the front TI will be shown on the VDU. The driver may choose to turn off the IR illuminator during tactical operations REVERSE MODE: VDU will automatically switch to show video feed from the rear far camera (default) or rear near camera |  | Test (T) | Acceptance Test Plan (ATP) Acceptance Test Report (ATR) |
| 1.2 | Operational State (vehicle power on) |  |  |  |
| 1.2.1 | The Video Display Unit (VCU) and Vehicle Display Unit (VDU) shall turn on when the vehicle is powered on. The VCU shall supply 12VDC power to the cameras, which shall also turn on. The VCU shall receive SDI video from the cameras and weapon video system and process the video signals and overlay any necessary on-screen display (OSD) information. The VCU shall then output the resultant video signal in HD-SDI format to the VDU and to the C2 computer systems. The IR illuminator shall be controlled by function buttons on the VDU. |  | Test (T) | Acceptance Test Plan (ATP) Acceptance Test Report (ATR) |
| 1.2.2 | The Vision System shall provide capacities such as night driving, situational awareness of the external environment and weapon vision to the driver and crew stations |  | Test (T) | Acceptance Test Plan (ATP) Acceptance Test Report (ATR) |
| 2 | FUNCTIONAL & PERFORMANCE REQUIREMENTS |  |  |  |
| 2.1 | The vision system shall automatically turn on when the vehicle is powered |  | Test (T) | Acceptance Test Plan (ATP) Acceptance Test Report (ATR) |
| 2.2 | Common video connector type and pin configuration shall be adopted to enable a common video data connection |  | Inspection (I) | Certificate of Conformance |
| 2.3 | The latency incurred for video processing, formatting and distribution shall not affect real time driving and weapon engagement performance (<150ms total) |  | Test (T) | Acceptance Test Plan (ATP) Acceptance Test Report (ATR) |
| 2.4 | For the VCU requirements, please refer to the "VCU" tab |  |  |  |
| 2.5 | For the VDU requirements, please refer to the "VDU" tab |  |  |  |
| 2.6 | For the TI requirements, please refer to the "TI" tab |  |  |  |
| 2.7 | For day cameras requirements, please refer to the "Day Cameras" tab |  |  |  |
| 2.8 | For the IR illuminator requirements, please refer to the "IR Illuminator" tab |  |  |  |
| 3 | EXTERNAL INTERFACE REQUIREMENTS |  |  |  |
| 3.1 | Mechanical |  |  |  |
| 3.1.1 | The vision system components housed within the vehicle (VDU and VCU) shall have an IP rating of IP65 minimum. The vision system components mounted on the exterior of the vehicle (TI, rear cameras, and IR illuminator) shall meet IP66 minimum. |  | Test (T) | MIL-STD 810G Test report |
| 3.2 | Electrical |  |  |  |
| 3.2.1 | The Vision System components shall be fully functional within the operational limits of MIL-STD 1275E |  | Test (T) | MIL-STD 1275E test report |
| 3.2.2 | The Vision System components shall have built-in protection against reverse polarity |  | Test (T) | ATP test report |
| 3.2.3 | All harnesses shall be of rugged construction and the wire terminations are to be made of high quality connectors with excellent resistance to damage from maintenance activity |  | Inspection (I) | Certificate of Conformance |
| 3.2.4 | Video output connector on the VCU shall conform to the Def-Stan 23-09 Ethernet Data Network Connector standardisation: Single channel Ethernet connectors at interface panel shall be of type MIL-DTL-38999 Series III Rev L Amdt 1(07/2009), D38999/XXαB35SA |  | Inspection (I) | Certificate of Conformance |
| 3.2.5 | All SDI video signal contacts shall use coaxial contacts. All HD-SDI contacts shall be size 12, using 603039 and 603052 for coaxial pins and sockets respectively. |  | Inspection (I) | Certificate of Conformance |
| 3.3 | Hydraulic |  |  |  |
| 3.3.1 | Not applicable |  |  |  |
| 3.4 | Software |  |  |  |
| 3.4.1 | The software shall be compliant to MIL-STD 882E |  | Test (T) | SWAT test report White box test report Black box test report |
| 3.4.2 | Any customization to the existing Off-the-shelf system to meet STELS requirements is preferred not to have any change to the system’s original software code. Change of parameters is allowable. |  | Test (T) |  |
| 4 | ENVIRONMENTAL REQUIREMENTS |  |  |  |
| 4.1 | All tests shall meet EQT and ESS requirements |  | Test (T) | MIL-STD 810G Test report |
| 4.1.1 | Refer to the following documents. |  |  |  |
| 5 | EMI & EMC and POWER REQUIREMENTS |  |  |  |
| 5.1 | Contractor shall fulfill the Ebox meet the MIL-STD-461F |  |  |  |
| 5.2 | Refer to EMI EMC requirements documents. |  | Test (T) | MIL-STD 461F Test report |
| 5.3 | Contractor shall fulfill the Ebox meet the MIL-STD-1275E |  |  |  |
| 5.4 | Refer to Power requirements documents. |  |  |  |
| 6 | PHYSICAL REQUIREMENTS |  |  |  |
| 6.1 | Hardware dimension(s) |  |  |  |
| 6.1.1 | Refer to each component's tab for its specifications |  | Inspection(I) | Certificate of Conformance |
| 6.2 | Hardware Weight(s) |  |  |  |
| 6.2.1 | Refer to each component's tab for its specifications |  | Inspection(I) | Certificate of Conformance |
| 6.3 | Paint |  |  |  |
| 6.3.1 | The components, which require painting, shall be prepared and painted in accordance with ME-TG-2020-004 |  | Inspection(I) | Certificate of Conformance |
| 6.3.2 | The painting shall be done using heat resistant and fire retardant type paint. |  | Inspection(I) | Certificate of Conformance |
| 6.3.3 | All slding parts and operation contact surfaces shall be free of paint. |  | Inspection(I) | Certificate of Conformance |
| 6.3.4 | The quality of the painting work shall meet the military or industrial requirements in terms of thickness, homogeneity and adhesiveness. |  | Inspection(I) | Certificate of Conformance |
| 7 | RESOURCE |  |  |  |
| 7.1 | The Vision System Components shall meet IPC 610 and IPC 620 Class 3 product acceptance criteria |  | Inspection(I) | Certificate of Conformance |
| 8 | SAFETY REQUIREMENTS |  |  |  |
| 8.1 | All material used in the construction of the Vision System Components shall be fire retardant |  | Inspection(I) | Certificate of Conformance |
| 9 | SYSTEM SECURITY |  |  |  |
| 9.1 | Refer to cybersecurity documents (Pending for Attachment) Conformance to ISO 28000 |  |  |  |
| 10 | RELIABILITY REQUIREMENTS |  |  |  |
| 10 | RELIABILITY & MAINTAINABILITY REQUIREMENTS |  |  |  |
| 10.1 | Refer to R&M documents. |  |  |  |
| 10.2 | Minimum Guaranteed MKBF/MTBF of Refer to MTBF figures on individual tabs |  |  |  |
| 11 | ILS REQUIREMENTS |  |  |  |
| 11.1 | Refer to ILS documents. |  |  |  |
| 12 | TRANSPORTABILITY |  |  |  |
| 12.1 | Refer to Supplier Quality Requirement document on Other Qualities (No 13.1) |  |  |  |
| 12.2 | Refer to ILS Requirement document on ILS Requirements (No 11.1) |  |  |  |
| 12.2 | Refer to ILS document |  |  |  |
| 13 | OTHER QUALITIES |  |  |  |
| 13.1 | Refer to Supplier Quality Requirement document. |  |  |  |
| 13.2 | The contractor shall provide a 12 months warrany against manufacturer defeats and workmanshop with the option of 2 yeas extension. |  |  |  |
| 13.3 | The contractor shall provide a Certificate of Conformance(COC) and test reports. |  |  |  |
| 13.4 | Unless otherwise specified in the contract/purchasing order or applicable drawing, the workmanship and acceptance requirements for the manufacturing of Printed Circuit Board (“PCB”) shall conform to the requirements specified in IPC-A-600, Class 3 product acceptance criteria. |  |  |  |
| 13.5 | Unless otherwise specified in the contract / purchasing order or applicable drawing, the workmanship and acceptance requirements for the manufacturing of electrical and electronic assemblies shall conform to the requirements specified in IPC-A-610, Class 3 product acceptance criteria. |  |  |  |
| 13.6 | Unless otherwise specified in the contract/purchasing order or applicable drawing, the workmanship and acceptance requirements for the manufacturing of cable and wire harness assemblies shall conform to the requirements in IPC/WHMA-A-620, Class 3 product acceptance criteria |  |  |  |
| 14 | Export Control |  |  |  |
| 14.1 | Supplier should indicate if Export Control (EUC / EUS) is required |  |  |  |